Supervisor: 	Dept: ______________________	Date: ___/___/___

###### When it’s time to use a Fire Extinguisher, just remember PASS!

PULL

Pull the Pin

AIM

Aim the nozzle or hose at

the base of the fire from the

recommended safe distance of 6-8ft.

SQUEEZE

Squeeze the operating

lever to discharge the fire

extinguishing agent.

SWEEP

Sweep the nozzle or

hose from side to side until

the fire is out. Move forward

or around the fire area as the

fire diminishes. Watch the

area in case of re-ignition.

*Visually Inspect Fire extinguishers monthly.  Is the pin intact? Is the gauge showing the correct pressure? Is the extinguisher visible and accessible?

* Recharge annually and maintain with local, state and national codes.